TITLE

Department of Information Engineering, Computer Science and Mathematics

University Of Study Of L’Aquila, Italy

Submitted to Professor Davide Di Ruscio

Course Titled Software Engineering for Internet of Things

| Date | 18/01/2022 |
| --- | --- |
| Team Name | MGM |

| Team Member | M. Number | Email Address |
| --- | --- | --- |
| Martina Nolletti | 286160 | martina.nolletti@student.univaq.it |
| Giordano Tinella |  | giordano.tinella@student.univaq.it |
| Michele Intrevado | 284789 | michele.intrevado@student.univaq.it |

Summary

1.	Introduction	3

2.	Technology Used/Simulated (System components)	3

3.	Architecture of the System	5

4.	Implementation of Sensors	6

4.1.	Node-Red Implementation	9

4.2.	Data Visualization Through Grafana	9

5.	Frontend Application	9

6.	Conclusion	9

# Introduction

Our system takes care of monitoring different values coming from solar panels and wind turbines.

Wind energy is primarily the use of wind turbines to generate electricity. It is a popular, sustainable, renewable energy source that has far less impact on the environment than the burning of fossil fuels. Wind farms consist of many individual wind turbines, which are connected to the electricity transmission grid. Wind energy is primarily the use of wind turbines to generate electricity. It is a popular, sustainable, renewable energy source that has far less impact on the environment than the burning of fossil fuels. Indeed, it aims to improve energy security and reduce the consumption of coal due to growing concerns about climate change and air pollution. Wind farms consist of many individual wind turbines connected to the electricity transmission grid. Through IoT connectivity, wind farm developers can effectively monitor the performance of wind turbines and related connected devices installed in remote locations without requiring any manual intervention.

Furthermore, solar panels also produce energy and by combining them with IoT technology it is possible to maximize energy production with a consequent reduction in costs and energy consumption.

It should also be considered that solar systems are constantly exposed to atmospheric agents and over the years, problems such as accumulation of dirt, voltage drops, and malfunctioning of the components can compromise the optimal performance of the system, affecting the effective production of power. For these reasons, the most recent models of systems are combined with monitoring systems based on IoT technology capable of detecting the entire system’s performance, identifying any malfunctions, and, consequently, increasing its efficiency and energy production.

Monitoring systems, therefore, represent a fundamental innovation to ensure optimal plant performance but can also offer other benefits. In addition to mapping energy production in real-time, they collect data and parameters regarding self-consumption and on-site exchange. In this way, we will always be aware of how much energy our system produces and how it is used.

The IoT applied to the solar panel also allows you to keep track of environmental parameters, such as the presence of rain, temperature, and solar radiation, allowing you to prevent any congestion or malfunctions of the system, for example by limiting the production of the system on rainy days or, conversely, maximizing the power on days characterized by low solar radiation.

# Technology Used/Simulated (System components)

To implement our application, we have used different types of technologies.

IoT sensors and components

Our system is simulated to give the vision of the use of different types of IoT sensors and components.

For solar panels you have the following sensors:

- Energy production sensor, to have the energy produced.

- Temperature sensor, to get the temperature.

- Light sensor, to get the brightness.

It also has one Shield Switch to protect the solar panels from weathering.

For wind turbines we have:

- Energy production sensor, to have the energy produced.

- Speed sensor, to have the speed.

- Vibration sensor, to have the vibrations of the blades when they are in operation.

It also has Run/Stop Switch to protect the wind turbines from weathering.

For the gateway we have:

- Weather sensor, to get information about the weather.

- Wind sensor, to get wind information.

MQTT

The protocol used by the Mosquitto broker is MQTT. We used this messaging protocol to get data from sensors. Then we loaded the data via python to send it to the other components where the data can be fetched and processed from.

Our published topics include:

- :solarPanel/0/light

- :solarPanel/0/temperature

- :solarPanel/0/production

- :windTurbine/0/production

- :windTurbine/0/speed

- :windTurbine/0/vibration

Node-RED

We used Node-RED to process the output data from MQTT. We also created the data flow through the use of Node-RED.

InfluxDB

InfluxDB is used to store the continuous flow of data coming and going through Node-RED. The main benefit of using InfluxDB is the ease with which data can be sorted and found.

Grafana (Dashboard)

We used Grafana to visualize and understand the data. The main benefit of Grafana that we found was that, in addition to providing better visualization, it provides a way to create multiple dashboards at once which allowed us to better manage the information.

# Architecture of the System

The diagram above shows a simple architecture of our system.

Our system architecture includes wind turbines and solar panels connected to sensors.

# Implementation of Sensors

We implemented the solar panel sensors by categorizing the values for day and night. We then define the topics to get the temperature, brightness and energy produced.

We implemented the sensors of the wind turbine defining the speed of the blades, the energy produced and the vibrations of the system when it is in action. We then define the topics to obtain the required values.

In addition, we have implemented our system with sensors for weather, wind and to get information about the day or night.

In main, the system and the client with MQTT are defined. The client connects to the correct address. Finally, the values of the sensors for the solar panels and for the wind turbines are simulated with a pause of 10 seconds.

# Node-Red Implementation

# Data Visualization Through Grafana

# Frontend Application

# Conclusion